# JMR Automation Tire Wash Tub - Owner's Manual

## PRODUCT OVERVIEW

The JMR Automation Cordless Tire Wash Tub is a mobile, efficient solution for cleaning dirt modified tires. Powered by a Milwaukee M18 or DeWalt 20V battery, it's ideal for both at-the-track and shop use.

## FEATURES

Cordless operation with Milwaukee M18 or DeWalt 20V battery (not included)

Simple On/Off switch that illuminates when active

6.5-gallon water reservoir with overflow control

Built-in stainless-steel brush continuously cleans rotating tire

Nozzle manifold for optional garden hose spray

Drain valve for easy emptying and flushing

Heavy-duty casters for full mobility

## QUICK START INSTRUCTIONS

Fill tub up to the drain hole with clean water.

Install battery into click on switch box.

Tilt the motor assembly back.

Install tire on hub and loosely spin on lug nuts.

Tilt the motor and tire assembly into tub.

Turn the switch “ON” to start rotation and cleaning.

- NOTE:	You may connect a garden hose to the nozzle

- manifold for more efficient cleaning.

- You may also use the supplied brush to help

- Clean the side walls, rim and tread of tire.

Turn the switch to “OFF” when tire is sufficiently cleaned.

Remove the tire from the system in the opposite order.

After cleaning tires, open the front valve to drain water.

Flush tub with clean water if needed and close valve.

## CLEANING & MAINTENANCE

Drain after every use.

Use only soap and low-pressure water for cleaning.

Use garden hose for thorough flushing.

Store in a dry environment.

Inspect brushes and drain regularly for debris.

## SAFETY NOTES

Use only Milwaukee M18 or DeWalt 20V batteries.

Do not operate with loose or damaged parts.

Keep away from children and pets.

Do not put hands between the tire and tub sides.

Do not use with pressure washing systems.